Controlled Failure of

Router-Based ROS 2 Middleware under Degraded Wireless Links

Alexandru GHEORGHITA

Institute of Solid Mechanics of the Romanian Academy (IMSAR)

Bucharest, Romania, alexandru.gheorghita@imsar.ro

# Abstract

Middleware benchmarks for ROS 2 report performance almost exclusively under nominal network conditions, and recent studies recommend the router-based rmw_zenoh for wireless deployments. We present a controlled degradation benchmark using tc netem that compares rmw_zenoh and rmw_cyclonedds in two environments with identical degradation parameters and an identical measurement protocol: software-in-the-loop (SIL, loopback, N=10) and hardware-in-the-loop (HIL, real Wi-Fi between two machines, N=5). The two environments disagree sharply. On loopback, Zenoh handles degradation comparably to CycloneDDS (1.3% vs. 1.8% sample loss under 200 ms latency with 50 ms jitter). On real Wi-Fi under the same emulated condition, Zenoh loses 96% of samples with a mean RTT of 6.2 s—more than 13x the RTT of CycloneDDS in the same representative run (469 ms, 2.7% loss; 14.7% aggregated)—although no packet loss is injected at all. The RTT structure indicates TCP retransmission and router-hop buffering triggered by the physical medium, a failure mode invisible on loopback. Our results (i) characterize regime-dependent failure modes of the two middlewares and (ii) carry a methodological warning: SIL-only validation would have certified the fragile middleware as safe. We release the benchmark, configurations, and data.

Index Terms: ROS 2, middleware, Zenoh, DDS, network degradation, hardware-in-the-loop, benchmark.

# 1. Introduction

Communication is a weak link of field robotics: disaster environments combine long relays, interference, and non-line-of-sight propagation into links with high latency and double-digit packet loss. ROS 2 delegates transport to interchangeable middleware implementations through the RMW abstraction layer [2], [19], and the recent rmw_zenoh [3] offers an alternative to the DDS family designed for constrained and wide-area networks. Existing comparisons, however, measure throughput and latency on healthy networks [5], leaving open the question that matters operationally: what does middleware choice cost when the network degrades?

A second, less visible question concerns how such an evaluation should be run. Simulation-based (SIL) benchmarks are cheap and reproducible; hardware-in-the-loop (HIL) benchmarks are costly. If the two agree, SIL suffices. We show that for this question they do not agree.

Contributions.

- A reproducible SIL + HIL testbed for controlled network degradation (tc netem) applied to ROS 2 middleware, with identical degradation parameters and measurement protocols in both environments.

- Comparative characterization of rmw_zenoh and rmw_cyclonedds failure modes under packet loss and latency on real Wi-Fi.

- The SIL-HIL divergence result: under latency-plus-jitter, Zenoh's collapse is induced by the physical medium, not by the protocol logic—simulation alone misses the dominant failure mode entirely.

- An architecture decision guide for middleware selection in wireless teleoperation, with a methodological caveat on SIL-only validation.

# 2. Background and Related Work

## 2.1 The RMW layer and the two middlewares

ROS 2 nodes talk to an abstract middleware interface (RMW); the concrete implementation is selected at runtime via RMW_IMPLEMENTATION [2]. rmw_cyclonedds implements the DDS/RTPS model: fully peer-to-peer, UDP-based, with decentralized discovery (SPDP/SEDP) that relies on periodic multicast announcements [4]. rmw_zenoh replaces DDS with the Zenoh protocol [12]: nodes hold TCP sessions to a local zenohd router, routers gossip discovery information among themselves, and multicast scouting is disabled by default [3]. The architectural contrast—brokered TCP versus peer-to-peer UDP—is the axis of this study.

Fig. 1. The RMW abstraction: identical application code runs over either middleware; only the transport stack changes.

## 2.2 Prior benchmarks

Early ROS 2 evaluations characterized DDS performance and executor overheads on healthy networks [8], [11]. Liang et al. [6] compared Zenoh, MQTT, Kafka, and DDS on throughput and latency, finding Zenoh competitive—but exclusively on healthy links. An edge-to-edge study [5] reports CycloneDDS is strongest on Ethernet and Zenoh is strongest on nominal Wi-Fi and 4G, attributing the latter to DDS multicast discovery struggling on wireless. Recent work documents DDS fragility under wireless packet loss [7], and loss-injection studies model DDS latency analytically [16]. Controlled netem injection on ROS 2 dates back to a 2016 evaluation of Connext and Fast RTPS [17], and a field study compares the middlewares on a physical mesh under real—but uncontrolled—degradation [18]; none of these include rmw_zenoh under parameterized impairment. To our knowledge, no prior work compares the two middlewares under controlled, parameterized degradation, nor contrasts SIL and HIL results for the same degradation matrix; this gap is the target of the present study.

## 2.3 Positioning

Our results do not contradict the nominal-Wi-Fi literature: they extend it into the degraded regime. The recommendation "Zenoh for wireless" holds for healthy links; we show it inverts once the physical link degrades.

# 3. Methodology

## 3.1 Testbed

SIL: Both endpoints and, for Zenoh, the router run on a single Linux machine over the loopback interface; degradation is applied to lo. This isolates protocol logic from all physical effects.

HIL: A laptop (client + Zenoh router) and a Raspberry Pi 4 (echo server + Zenoh router) communicate over a real consumer Wi-Fi network; degradation is applied symmetrically with netem on the wireless interface of both machines, so both middlewares experience identical network physics on top of the real medium.

Both environments run ROS 2 Jazzy with unmodified application code; the middleware is switched only via RMW_IMPLEMENTATION. The laptop runs rmw_zenoh 0.2.9 and rmw_cyclonedds 2.2.3 (Eclipse Cyclone DDS 0.10.5) on Linux kernel 6.17. The Zenoh mesh uses one router per machine (listening on all interfaces, with an explicit unicast connection between the two), since multicast scouting is disabled by default [3].

Fig. 2. SIL architecture: single host, loopback, netem on lo.

Fig. 3. HIL architecture: two hosts over real Wi-Fi, symmetric netem, and one Zenoh router per host with explicit unicast connection.

## 3.2. The model of degradation

Degradation is induced with TC netem [9], [10]. Table I lists the eight conditions. The netem qdisc is attached in every condition, including ideal (delay 0 ms, jitter 0 ms, loss 0%), so all conditions run under the same queueing discipline and differ only in the emulation parameters. Loss uses netem's Bernoulli i.i.d. model (each packet dropped independently with probability p). Jitter is applied without a distribution table; in this configuration, the netem kernel implementation draws the per-packet delay uniformly.

bounding the delay to [DELAY - JITTER, DELAY + JITTER]. We note a documented discrepancy: the tc-netem manual page states that the default distribution is normal [9], while the kernel source (tabledist() in sch_netem.c) applies a uniform fallback when no distribution table is loaded; we report the distribution the kernel actually applied. We deliberately use the memoryless loss model for comparability with prior work; bursty models (Gilbert-Elliott) are future work (Sec. 7).

| Condition | Delay (ms) | Jitter (ms) | Loss (%) |
| --- | --- | --- | --- |
| ideal | 0 | 0 | 0 |
| loss_5 | 0 | 0 | 5 |
| loss_15 | 0 | 0 | 15 |
| loss_20 | 0 | 0 | 20 |
| loss_25 | 0 | 0 | 25 |
| loss_30 | 0 | 0 | 30 |
| lat200_jit50 | 200 | 50 | 0 |
| lat200_l15 | 200 | 50 | 15 |

Table I. Netem conditions (identical in SIL and HIL; source: bench_core.py).

## 3.3 Metrics and protocol

A client publishes timestamped payloads echoed by a server; RTT uses a single clock. Per run, we record RTT percentiles (p50/p95/p99), mean, min, max, and sample loss = 1 - received/sent, where a sample counts as lost if its echo has not returned by the end of the run window (run duration plus a 1.5 s grace period). Each run sends 989 samples at 50 Hz for 20 s per payload class (64 B command, 4 KB telemetry, 64 KB map/image). Conditions are repeated N=10 times (SIL) and N=5 times (HIL); condition-level figures aggregate per-repetition summaries.

## 3.4 Reproducibility

Benchmark code, router configurations (JSON5), netem manifests, and all raw campaign data are publicly available in the project repository (github.com/alexandru-tech-web/ROS2, tag c1-paper-v3.4).

# 4. Results

Unless stated otherwise, figures refer to the 4 KB payload class.

## 4.1 Baseline (ideal)

On loopback both middlewares are equivalent: mean RTT of 1.1-1.4 ms and 0% loss. On real Wi-Fi, both remain clean at 64 B and 4 KB (0% loss), with a mean RTT of 11-13 ms at 4 KB. At 64 KB on Wi-Fi, however, Zenoh loses 58% of samples with no degradation injected, while CycloneDDS loses none of the samples it sends (Fig. 4): with large payloads on a bandwidth-limited link, CycloneDDS throttles the publisher to ~364 of the 989 intended samples, whereas Zenoh keeps sending all 989 without backpressure. The two stacks fail differently at saturation: CycloneDDS sheds rate, Zenoh sheds samples. The effect reproduced identically across three separate campaign runs, so we report it as an architectural property, not an artifact.

Fig. 4. Effect of payload size on sample loss (HIL Wi-Fi, N=5): ideal condition (top) and 15% injected loss (bottom).

## 4.2 SIL: the protocol in isolation

Table II (SIL columns) shows the loopback picture (Fig. 5, top). Under pure loss Zenoh degrades earlier than CycloneDDS (8.5% vs. 1.4% sample loss at 15% injected loss), but both remain functional, and under pure latency plus jitter, the two are practically indistinguishable (1.3% vs. 1.8%). Judging on SIL alone, Zenoh appears a reasonable choice for degraded links.

## 4.3 HIL Wi-Fi: the real medium

The HIL columns of Table II tell a different story (Fig. 5, bottom). Under-injected loss Zenoh collapses roughly 3x harder than CycloneDDS at every level (61.6% vs. 20.8% at loss_15) and saturates near-total loss from loss_20 onward. Most strikingly, under lat200_jit50—200 ms delay, 50 ms jitter, and zero injected loss, Zenoh loses 96.3% of samples while CycloneDDS loses 14.7%.

| Condition | SIL CDDS | SIL Zenoh | HIL CDDS | HIL Zenoh |
| --- | --- | --- | --- | --- |
| ideal | 0.0 | 0.0 | 0.0 | 0.0 |
| loss_5 | 0.0 | 0.0 | 0.0 | 0.0 |
| loss_15 | 1.4 | 8.5 | 20.8 | 61.6 |
| loss_20 | 7.7 | 16.9 | 53.5 | 93.7 |
| loss_25 | 26.5 | 34.1 | 70.4 | 98.8 |
| loss_30 | 41.0 | 57.8 | 80.5 | 99.2 |
| lat200_jit50 | 1.8 | 1.3 | 14.7 | 96.3 |
| lat200_l15 | 36.0 | 20.8 | 72.4 | 100.0 |

Table II. Transport sample loss, 4 KB payload. SIL: N=10; HIL Wi-Fi: N= 5. Source: campaign summaries, 2026-07-01.

Fig. 5. Transport sample loss per netem condition and middleware (4 KB payload): SIL loopback, N=10 (top) vs. HIL real Wi-Fi, N=5 (bottom). Identical injected parameters in both environments: data as in Table II.

## 4.4 The SIL-HIL divergence

The same middleware, under the same netem parameters and the same measurement protocol, behaves radically differently depending only on the physical medium: at lat200_jit50, Zenoh goes from 1.3% loss (SIL) to 96.3% (HIL). CycloneDDS shifts too (1.8% to 14.7%) but remains functional. The divergence, not either environment alone, is the central result.

Fig. 6. The SIL-HIL divergence at lat200_jit50 (200 ms delay, 50 ms jitter, no injected loss): identical code and netem parameters, different physical medium. Data: Table II.

## 4.5 Failure mechanism: what the RTT structure shows

Table III gives the aggregated p95 RTT at lat200_jit50 (per-condition p95: Fig. 7); the per-run detail is sharper. In a representative HIL run, CycloneDDS returns 962/989 samples with a mean RTT of 469 ms and a tight distribution (min. 402 ms, max. 543 ms)—exactly the injected 200 ms each way plus Wi-Fi overhead. Zenoh returns 184/989 samples with mean RTT 6,185 ms (min 1,591, max 7,872 ms). In the other four of the five repetitions under this condition, Zenoh returned zero samples; the run shown is the surviving best case. The excess of ~5.7 s over the injected latency cannot originate in the channel; its structure is consistent with TCP retransmission timeouts compounding across the two router hops (node-router-router-node) and with router-side buffering. Because the injected jitter is uniformly bounded (Sec. 3.2), the emulated channel adds at most 250 ms per direction; CycloneDDS's maximum RTT of 543 ms in the same run bounds the medium's typical contribution. The multi-second Zenoh RTTs therefore cannot be explained by the emulated channel and must originate in the transport stack. On loopback the same condition yields a clean Zenoh RTT (mean 427 ms, max 636 ms, 977/989 received): without the physical medium's contention and radio jitter, the cascade is never triggered. We state this as an inference from the RTT structure; direct protocol-level instrumentation is future work (Sec. 7).

| Env. | RMW | RTT p95 (ms) | Loss (%) |
| --- | --- | --- | --- |
| SIL | CycloneDDS | 484 | 1.8 |
| SIL | Zenoh | 476 | 1.3 |
| HIL | CycloneDDS | 635 | 14.7 |
| HIL | Zenoh | 7,697 | 96.3 |

Table III. RTT p95 (ms) and loss at lat200_jit50, 4 KB payload, aggregated over repetitions.

Fig. 7. Per-condition p95 RTT on HIL Wi-Fi (log scale, 4 KB payload, N=5); received=0 marked where no samples were returned. p95 is computed over delivered samples only; under near-total loss, it reflects a shrinking, survivorship-biased subset (hence loss_30 < loss_25 for Zenoh).

# 5. Discussion

## 5.1 Why the environments disagree

SIL isolates the protocol logic. Netem delay is “clean” – added in the kernel, no contention, collisions or radio level jitter. On that substrate Zenoh's TCP sessions never enter sustained retransmission. The real Wi-Fi medium adds 802.11 contention, retries, and rate adaptation on top of the injected impairment; combined latency and jitter push TCP round-trip-time estimators and retransmission timers into a regime where windows collapse and retransmissions compound at each router hop—the classic failure mode of TCP over lossy wireless links [13], amplified by queue buildup along the path [14]. CycloneDDS over UDP has no such machinery: samples are late or lost but never amplified into multi-second queues.

## 5.2 Methodological implications

Had we run only the (cheaper, more convenient) SIL campaign, Zenoh would have appeared safe for degraded links. The HIL campaign inverts the conclusion. For middleware selection on wireless robots, SIL-only validation can produce false confidence; the physical medium is not a nuisance parameter but the trigger of the dominant failure mode.

## 5.3 Architecture decision guide

On healthy Wi-Fi with small-to-moderate payloads, the literature's recommendation of Zenoh stands [5]. On degraded wireless links—the SAR regime—our data favor CycloneDDS: graceful, roughly proportional degradation instead of cliff-edge collapse. Where large payloads must cross a bandwidth-limited link, CycloneDDS additionally avoided the ideal-condition losses observed for Zenoh (Sec. 4.1).

# 6. Limitations

(1) A single hardware testbed (one laptop, one Raspberry Pi 4, one consumer access point); claims are scoped to this setup. (2) Memoryless Bernoulli loss and uniformly distributed, bounded jitter (Sec. 3.2); bursty loss (Gilbert-Elliott) is not modeled. (3) Stock middleware configurations and tuned Zenoh (TCP buffers, QoS, and batching) might shift the boundary. (4) N=10 (SIL) vs. N=5 (HIL) repetitions. (5) The failure mechanism is inferred from RTT structure, not from protocol-level instrumentation. (6) Netem's realized jitter has been reported to undershoot its nominal value [15]; we did not verify the realized jitter distribution on our kernels, so the effective jitter at lat200_jit50 may be below the nominal 50 ms.

# 7. Future Work

# Burst loss models (Gilbert-Elliott) to verify whether correlated loss exacerbates router-level failure; protocol-level instrumentation (packet capture, router logs) in the lat200_jit50 scenario to directly confirm the retransmission cascade; Zenoh optimization as a study on mitigation strategies; and dynamic RMW selection based on measured link status, feeding into the SAR swarm demonstrator derived from the experimental analysis.

# 8. Conclusion

# Under controlled degradation conditions with identical parameters, the two ROS 2 transport communication technologies diverge only when the physical environment is real: on loopback, Zenoh matches CycloneDDS, but on real Wi-Fi, it collapses—most dramatically regarding latency and jitter, even without injected packet loss (96% vs. 15% sample loss; 12x p95 RTT). The choice of middleware for wireless robots depends on the operating regime, and that regime cannot be fully investigated through simulation alone: ​​the environment is the mechanism that ensures a correct understanding of communication technologies and determines which technology suits the situations encountered.

# References

[1] Casper J, Murphy RR. Human-robot interactions during the robot-assisted urban search and rescue response at the World Trade Center. IEEE Trans Syst Man Cybern B Cybern. 2003;33(3):367-85. doi: 10.1109/TSMCB.2003.811794. PMID: 18238185.

[2] Different ROS 2 middleware vendors. (2026). https://docs.ros.org/en/humble/Concepts/Intermediate/About-Different-Middleware-Vendors.html. Retrieved May 11, 2026, from https://docs.ros.org/en/humble/Concepts/Intermediate/About-Different-Middleware-Vendors.html

[3] Ros. (n.d.). rmw_zenoh/docs/design.md at rolling · ros2/rmw_zenoh. GitHub. https://github.com/ros2/rmw_zenoh/blob/rolling/docs/design.md

[4] Committers, E. C. D. (n.d.). Eclipse Cyclone DDS - Documentation. https://cyclonedds.io/docs/

[5] Zhang, J., Yu, X., Ha, S., Queralta, J. P., & Westerlund, T. (2024). Comparison of middlewares in Edge-to-Edge and Edge-to-Cloud communication for distributed ROS 2 systems. Journal of Intelligent & Robotic Systems, 110(4). https://doi.org/10.1007/s10846-024-02187-z

[6] Liang, W., Yuan, Y., & Lin, H. (2023, March 16). A performance study on the throughput and latency of Zenoh, MQTT, Kafka, and DDS. arXiv.org. https://arxiv.org/abs/2303.09419

[7] Lee, S., Kim, T., Chae, J., & Park, K. (2025, August 15). Optimizing ROS 2 communication for wireless robotic systems. arXiv.org. https://arxiv.org/abs/2508.11366

[8] Yuya Maruyama, Shinpei Kato, and Takuya Azumi. 2016. Exploring the performance of ROS2. In Proceedings of the 13th International Conference on Embedded Software (EMSOFT '16). Association for Computing Machinery, New York, NY, USA, Article 5, 1–10. https://doi.org/10.1145/2968478.2968502

[9] tc-netem(8) - Linux manual page. (n.d.). https://man7.org/linux/man-pages/man8/tc-netem.8.html

[10]Hemminger, S. & Open Source Development Lab. (2005). Network Emulation with NetEm. https://www.rationali.st/jittertrap-baby-steps-in-dsp/netem-shemminger.pdf

[11] Kronauer, T., Pohlmann, J., Matthe, M., Smejkal, T., & Fettweis, G. (2021, January 6). Latency analysis of ROS2 Multi-Node Systems. arXiv.org. https://arxiv.org/abs/2101.02074

[12] A. Corsaro et al., "Zenoh: Unifying communication, storage and computation from the cloud to the microcontroller," in Proc. 26th Euromicro Conf. Digital System Design (DSD), 2023, pp. 422-428. doi:10.1109/DSD60849.2023.00065.

[13]Hari Balakrishnan, Venkata N. Padmanabhan, Srinivasan Seshan, and Randy H. Katz. 1997. A comparison of mechanisms for improving TCP performance over wireless links. IEEE/ACM Trans. Netw. 5, 6 (Dec. 1997), 756–769. https://doi.org/10.1109/90.650137

[14] Jim Gettys and Kathleen Nichols. 2011. Bufferbloat: Dark Buffers in the Internet: Networks without effective AQM may again be vulnerable to congestion collapse. Queue 9, 11 (November 2011), 40–54. https://doi.org/10.1145/2063166.2071893

[15] Jurgelionis, A., Laulajainen, J., Hirvonen, M., & Wang, A.I. (2011). An Empirical Study of NetEm Network Emulation Functionalities. 2011 Proceedings of 20th International Conference on Computer Communications and Networks (ICCCN), 1-6.

[16] Lee, S., Park, H., Chae, J., & Park, K. (2025, August 14). Probabilistic Latency Analysis of the Data Distribution Service in ROS 2. arXiv.org. https://arxiv.org/abs/2508.10413

[17] A. Dabrowski, R. Kozik, and M. Macias, "Evaluating the

resilience of ROS2 communication layer," presented at ROSCon 2016,

Seoul, South Korea, Oct. 2016. [Online]. Available:

https://roscon.ros.org/2016/presentations/rafal.kozik-ros2evaluation.pdf.

Accessed: Jul. 11, 2026.

[18] Chovet, L. P., Garcia, G. M., Bera, A., Richard, A., Yoshida, K., & Olivares-Mendez, M. A. (2024, July 3). Performance comparison of ROS2 middlewares for multi-robot mesh networks in planetary exploration. arXiv.org. https://arxiv.org/abs/2407.03091

[19] Macenski, S., Foote, T., Gerkey, B., Lalancette, C., & Woodall, W. (2022). Robot Operating System 2: Design, architecture, and uses in the wild. Science Robotics, 7(66), eabm6074. https://doi.org/10.1126/scirobotics.abm6074